TRUENAS BUILD PROJECT

# GA-EP45-DS4P
Maximum-Use NAS Plan

What the project is, how the parts fit together, and the order to build it

| Motherboard | Gigabyte GA-EP45-DS4P (rev. 1.0) |
| --- | --- |
| Processor | Intel Core 2 Quad |
| Memory | Currently about 7.5-8 GB DDR2; target 16 GB |
| Operating system | TrueNAS SCALE |
| Main objective | Use IDE, SATA, and PCIe/NVMe storage while maximizing practical network performance |

Important reality: two teamed 1 GbE ports do not normally give one computer a 240-300 MB/s file copy. They mainly improve total traffic across multiple clients. A 10 GbE upgrade is the clean path to 300 MB/s from one client.

# 1. Project Summary

This project converts the GA-EP45-DS4P system into a multi-tier TrueNAS server. The design separates slow archival disks, the main SATA storage pool, and a faster NVMe pool instead of mixing all drives into one ZFS pool.

ArchivePool: two IDE/PATA hard drives in a mirror for cold storage and backups.

MainPool: six SATA drives on the motherboard, configured as three mirrors or RAIDZ2.

FastPool: four NVMe drives on a PCIe adapter, configured as two mirrors striped together.

Auxiliary NVMe: one small NVMe intended for specialized log/cache use only after testing.

Network: use the two onboard Gigabit ports for redundancy and multi-client throughput, or add 10 GbE for higher single-client speed.

## What success looks like

| Goal | Practical result | Main requirement |
| --- | --- | --- |
| Reliable NAS storage | Separate pools with ZFS redundancy | Healthy drives, backups, SMART tests and scrubs |
| Use all onboard storage | 2 IDE + 6 SATA devices | Enough power, cables, cooling and drive bays |
| Fast local pool | 4 NVMe devices if adapter/slot support works | PCIe bifurcation or a switched NVMe adapter |
| About 240 MB/s total network traffic | Possible across multiple clients using 2x1 GbE LACP | Managed switch with LACP |
| 300 MB/s or more to one client | Use 10 GbE | 10 GbE NIC, compatible switch/direct link and fast pool |

# 2. Hardware Specifications and Planned Use

| Component | Project specification | How it will be used |
| --- | --- | --- |
| Motherboard | Gigabyte GA-EP45-DS4P rev. 1.0 | Legacy BIOS; no native NVMe boot |
| Chipset | Intel P45 + ICH10R | Use AHCI; do not create motherboard RAID |
| CPU | Intel Core 2 Quad | Suitable for basic NAS work; limited for heavy apps/VMs |
| Memory | About 7.5-8 GB DDR2; board plan targets up to 16 GB | Upgrade to 16 GB before buying cache drives if possible |
| SATA | 6 x SATA II, 3 Gb/s | MainPool and/or dedicated boot device |
| IDE/PATA | 1 connector, up to 2 devices | ArchivePool only |
| Networking | 2 x 1 GbE onboard | LACP gives redundancy and aggregate bandwidth |
| PCIe storage | Planned 4-NVMe card plus single-NVMe card | Compatibility must be proven before buying all NVMe drives |
| Power supply | Plan calls for 600-700 W quality PSU | Check connector count and 12 V capacity |

## Critical PCIe/NVMe compatibility checkpoint

A passive four-M.2 NVMe adapter normally needs the motherboard BIOS to split one PCIe slot into four separate links (PCIe bifurcation). This older board may not provide that feature. A passive quad card may detect only one drive or none. Before purchasing four NVMe drives, test the exact adapter. A PCIe-switch-based adapter can present multiple NVMe drives without motherboard bifurcation, but it is more expensive and must have TrueNAS/Linux support.

The project documents support a practical design of four NVMe drives on one card plus one single NVMe drive. The earlier idea of two four-drive cards (eight NVMe drives) is not treated as confirmed.

# 3. Final Storage Layout

| Pool / device | Connection | Drive count | ZFS layout | Purpose | Key warning |
| --- | --- | --- | --- | --- | --- |
| Boot device | SATA or USB-attached SSD | 1 | Not part of data pools | TrueNAS operating system | Do not install TrueNAS on a data-pool disk |
| ArchivePool | IDE/PATA | 2 | Mirror | Cold backups and archives | Old IDE disks are slow and may be unreliable |
| MainPool | 6 onboard SATA II | 6 | 3 mirrors striped, or RAIDZ2 | Primary SMB/NFS storage | All six ports are consumed if boot is elsewhere |
| FastPool | PCIe 2.0 adapter | 4 | 2 mirrors striped | VMs, scratch, fast shares | Requires working multi-NVMe adapter |
| Log/cache NVMe | PCIe x4 single adapter | 1 | Special VDEV only | Possible SLOG or L2ARC | Not a replacement for RAM; do not use blindly |

## Recommended MainPool choice

For this older platform, three mirrored pairs are the easiest high-performance arrangement and allow one drive in each pair to fail without losing the pool. RAIDZ2 gives two-disk parity and better usable capacity consistency, but generally has slower small writes and is less flexible to expand. Use equal-capacity drives whenever possible; ZFS uses the smallest drive size within each vdev.

## Do not combine these tiers

Do not place IDE drives in the same vdev as SATA SSDs or NVMe drives.

Do not stripe cache or log devices expecting them to protect data.

Do not count ArchivePool replication as the only backup if the entire machine could be lost.

# 4. Network Plan and Realistic Speed

The motherboard has two onboard Gigabit Ethernet ports. Teaming them with LACP creates one logical interface, but traffic is distributed by connection. A normal single SMB file-copy connection usually remains limited to one 1 GbE path.

| Network setup | Single-client expectation | Total expectation | Best use |
| --- | --- | --- | --- |
| One 1 GbE port | About 110-120 MB/s | About 110-120 MB/s | Simple home NAS |
| Two 1 GbE ports with LACP | Usually about 110-120 MB/s per connection | Up to about 220-240 MB/s across multiple clients | Redundancy and simultaneous users |
| 10 GbE NIC | Potentially 300 MB/s to about 1 GB/s, depending on storage/CPU | Same order of magnitude | One fast workstation or many users |

## LACP setup

1. Prepare the switch

Use a managed switch that supports IEEE 802.3ad LACP. Put the two switch ports connected to the NAS into the same LACP group.

2. Create the link aggregation

In TrueNAS, create a link aggregation using both onboard NICs and select LACP.

3. Assign the IP correctly

Assign the NAS static IP address to the new aggregate interface, not to the two physical interfaces.

4. Test failure and load

Disconnect one cable to confirm failover, then test with two client computers copying at the same time.

For a 300 MB/s target from one PC, budget for a supported 10 GbE card such as an Intel X520-class or Mellanox ConnectX-3-class adapter, plus SFP+ DAC or compatible switching.

# 5. Build Order: Physical Hardware

1. Inventory and label all parts

Record every drive model, serial number and capacity. Label cables and drive bays before installation.

2. Upgrade memory first

Install matched DDR2 modules and run a full memory test. The preferred target in the project is 16 GB.

3. Install CPU cooling and case airflow

Clean the heatsink, apply thermal compound if needed, and make sure air moves over the chipset, SATA drives and NVMe adapters.

4. Install the power supply

Use a quality 600-700 W unit with enough SATA/Molex connectors. Avoid long chains of cheap splitters.

5. Connect the IDE pair

Set one IDE drive as master and the other as slave, or use cable-select consistently. Put them on the single PATA cable.

6. Connect the six SATA data drives

Attach each drive directly to an ICH10R SATA port. Use AHCI, not motherboard RAID.

7. Install and test the quad-NVMe adapter

Install only one NVMe first and confirm detection. Then add the remaining drives one at a time.

8. Install the single-NVMe adapter

Use it only after the main system is stable. Its final role depends on workload testing.

9. Install a dedicated boot device

Use a separate small SSD. Because all six SATA ports are planned for MainPool, consider a reliable USB-to-SATA SSD arrangement or reserve one SATA port and reduce MainPool drive count.

10. Complete cable and thermal checks

Confirm every fan spins, every drive receives power, and no cable obstructs airflow.

## Boot-drive conflict to resolve

The project wants all six onboard SATA ports used by MainPool and also recommends a dedicated SATA boot SSD. Both cannot happen at the same time without another controller or an external/USB-attached boot SSD. Choose one of these: (A) five SATA data drives plus one SATA boot SSD; (B) six SATA data drives plus a reliable USB-attached SSD boot device; or (C) add a supported SATA/HBA controller.

# 6. BIOS and TrueNAS Installation

| BIOS item | Setting |
| --- | --- |
| SATA mode | AHCI |
| Motherboard RAID | Disabled / not used |
| Boot mode | Legacy BIOS |
| Boot priority | Installer USB first, then dedicated boot SSD |
| Unused devices | Disable only after the system is stable |
| PCIe generation | Use Gen2/Auto as supported |

## Installation procedure

1. Create the installer

Write the TrueNAS SCALE installer image to a USB flash drive.

2. Install to the dedicated boot device

Carefully select only the boot SSD. Do not select any disk planned for a storage pool.

3. Boot and record the management address

Connect one network cable initially and open the displayed web address from another computer.

4. Set administrative access

Use a strong password, configure the correct time zone and apply updates only after preserving the current configuration backup.

5. Verify all hardware

Check that the expected SATA, IDE and NVMe devices are visible before creating any pool.

# 7. Create the Pools and Shares

1. Create ArchivePool

Select the two IDE drives and create one mirror vdev. Use it for replication targets and cold data, not active workloads.

2. Create MainPool

Select the six SATA drives. Choose either three mirrors striped together or RAIDZ2. Confirm every serial number before clicking Create.

3. Create FastPool

Only after all four NVMe drives remain stable under load, create two mirrored pairs and stripe the mirrors.

4. Create datasets

Create separate datasets for general files, backups, media and any VM/database data.

5. Enable compression

Use LZ4 compression unless a specific workload requires something else.

6. Create SMB shares

Create a share for each intended dataset, set permissions, and test from one client before adding more users.

7. Set snapshots

Schedule regular snapshots on MainPool and FastPool.

8. Set replication

Replicate important MainPool datasets to ArchivePool, while recognizing that an in-box copy is not protection from theft, fire or total system failure.

## Dataset starting points

| Dataset type | Record size starting point | Compression |
| --- | --- | --- |
| General files / media | 128K | LZ4 |
| VMs / databases | 16K-32K | LZ4 |
| Backups / large sequential files | 128K or larger after testing | LZ4 |

# 8. NVMe Cache, SLOG and Swap: Use With Caution

The original project proposes splitting a 128 GB NVMe device into SLOG, swap and L2ARC partitions. Treat this as an experimental option, not a required step. It adds complexity and creates multiple important functions on one device.

| Feature | What it actually helps | When it helps | Main caution |
| --- | --- | --- | --- |
| SLOG | Synchronous write latency | NFS, databases, VMs or applications issuing sync writes | Needs high endurance and preferably power-loss protection; it does not speed ordinary asynchronous copies |
| L2ARC | Repeated reads that do not fit in RAM | A proven hot working set after ARC is already effective | Consumes RAM for metadata and may hurt an 8 GB system |
| Swap | Prevents immediate failure during memory pressure | Emergency fallback only | NVMe is not RAM; swapping heavily makes the server slow and increases wear |

## Recommended decision

First operate the server without L2ARC or a special SLOG. Measure memory use, ARC hit rate, synchronous-write workload and network speed. Upgrade to 16 GB RAM before using NVMe as a substitute for memory. Add a SLOG only if testing proves the workload is dominated by synchronous writes. Add L2ARC only if repeated reads miss ARC and the system still has adequate RAM.

# 9. Testing and Acceptance Checklist

☐ Memtest completes without errors.

☐ All expected drives appear with the correct model and serial number.

☐ No pool uses disks from different performance tiers in the same vdev.

☐ SMART short and long tests complete on every SATA/IDE drive.

☐ NVMe temperatures remain controlled during sustained writes.

☐ MainPool and FastPool complete a scrub without errors.

☐ One 1 GbE connection reaches roughly 110-120 MB/s with large sequential files.

☐ Two clients can run simultaneously across LACP without one cable becoming a single point of failure.

☐ Snapshots are visible and a test file can be restored.

☐ Replication to ArchivePool completes and a test restore succeeds.

☐ TrueNAS configuration is exported and stored somewhere outside the NAS.

## Suggested benchmark sequence

1. Test the network only

Use iperf3 between a client and TrueNAS. This separates network limitations from disk limitations.

2. Test each pool locally

Measure sequential reads/writes while watching CPU use, drive temperatures and errors.

3. Test SMB

Copy one large file, then many small files. Record results separately.

4. Test multi-client LACP

Run two or more clients at the same time. Aggregate throughput matters more than one-client speed.

5. Test recovery

Disconnect one mirror member only in a controlled test environment, confirm the degraded state is detected, and follow the documented replacement process.

# 10. Ongoing Maintenance

| Frequency | Task |
| --- | --- |
| Daily/continuous | Review alerts; confirm pools are online; watch NVMe and drive temperatures. |
| Weekly | Review SMART attributes and failed login/security alerts. |
| Monthly | Run ZFS scrubs; verify snapshot and replication jobs. |
| Quarterly | Perform a test restore; export a fresh TrueNAS configuration backup; clean dust filters. |
| Before every major update | Export configuration, confirm backups, read release notes, and avoid updating during an active resilver or scrub. |

# 11. Final Recommended Configuration

| Area | Recommendation |
| --- | --- |
| CPU | Core 2 Quad, preferably the strongest compatible model already available |
| RAM | 16 GB DDR2 preferred; 8 GB is the minimum project starting point |
| Boot | Dedicated SSD, outside the six-drive MainPool if possible |
| ArchivePool | 2 IDE HDDs, mirror |
| MainPool | 6 SATA drives, three mirrors striped or RAIDZ2 |
| FastPool | 4 NVMe drives, two mirrors striped, only after adapter compatibility is proven |
| Auxiliary NVMe | Leave unused initially; later assign as SLOG or L2ARC only if measurements justify it |
| Network | 2x1 GbE LACP for redundancy/multiple clients; 10 GbE for 300 MB/s single-client target |
| Cooling | Direct airflow over NVMe cards and adequate drive-bay airflow |
| Backups | Snapshots + replication + at least one copy outside this machine |

# 12. Project Risks to Resolve Before Buying Parts

Confirm whether the selected quad-NVMe card contains its own PCIe switch or requires motherboard bifurcation.

Resolve where the TrueNAS boot SSD will connect if all six motherboard SATA ports are reserved for MainPool.

Confirm that all IDE drives are healthy enough to trust even as a mirrored archive pool.

Confirm the managed switch supports LACP before planning on dual-port aggregation.

Confirm case space, power connectors and airflow for the total drive count.

Decide whether the real target is 240 MB/s across several clients or 300 MB/s from one client; the latter needs 10 GbE.